| DEFINITIONS |  |
| --- | --- |
| BALANCE | To remove colour cast & ensure blacks, whites and greys are free of colour |
| CHROMA (C) | Chrominance (Colour) |
| CLIPPING | Washed out Highlights (solid white block at head of Waveform) |
| CRUSHING | Blocked-up Blacks (shadows) |
| GAIN | Increase (or decrease) Luminance (usually by a multiple) |
| GAMMA | - Mid-tones or mid-range tones of a picture - The curve or steepness of a transition from Black to White |
| HUE | The colour: Each Hue has 256 values (0 = fully transparent; 256 = fully opaque) |
| HSB or HSL | Hue, Saturation and Brightness (Luminance) |
| LIFT/PEDESTAL/SETUP | Baseline for indicating Black |
| LUMINANCE (Y) | Brightness or transparency |
| PRIMARY COLOURS | Green, Red & Dark Blue |
| RGB PARADE MODE | In Waveform: shows individual levels of Red, Green & Blue Channels (Primaries) |
| SATURATION | The Intensity, Brilliance or Brightness of a colour |
| SECONDARY COLOURS | Yellow. Purple & Cyan Blue |

MEASURING TOOLS

| TO OPEN ALL SCOPES: Ctrl + Alt + 2 |  |  |  |
| --- | --- | --- | --- |
| VECTORSCOPE | WAVEFORM | HISTOGRAM | RGB PARADE |
| Displays Chrominance (Colour – C) and Hue: Plots Hue & Saturation on colour wheel Saturation of colour measured by how far it extends from centre with neutral (B&W and Grey) as dot at centre Hue indicated by position around circle perimeter (i.e. angle from centre point) Vector is location of coordinates on Scope | Displays luminance (Y) with brightest at top and darkest at bottom (to include Chroma choose Composite): Reads frame from left to right Lowest 0 IRE Highest 100 IRE for luminance only or 110 IRE for luminance and chroma Low Pass = luminance only Flat (Y/C mode) = luminance + chroma Pure white, black & grey means Red, Green & Blue Channels all match regardless of level | Indicates Tonal Range (Black on Left, Highlights on Right) AND number of pixels in each tonal value: (higher the peak the greater the luminance) (i.e. monitors colour levels AND contrast). Number of pixels at cursor shown in top right corner Mean Values = average intensity of all pixels & Standard Deviation = average percentage by which pixels vary from mean 1. Pixels concentrated to left or right = underexposed (dark) or overexposed (clipped) 2. Pixels banded=sharp changes between shades 3. Pixels in U-shape=high contrast 4. Limited range of pixels=faded image 5. Pixels evenly distributed or bell-shaped = balanced | Displays Waveforms for RGB components of video Shows RGB values from 0 to 255 on vertical axis (Keep within range 16 to 234) |

COLOUR CORRECTION BASICS

| PRIMARY COLOUR CORRECTION | Applies to all of frame and event |
| --- | --- |
| SECONDARY COLOUR CORRECTION | Alters single colours independently – can stack several adjustments |
| SECONDARY COLOUR | Lies between two primary colours on Vectorscope or ColourWheel |
| COMPLEMENTARY COLOUR | Lies directly opposite on Vectorscope or Colour Wheel |
| TO REDUCE A COLOUR | Increase its complementary or decrease the other two primaries |
| TO INCREASE A COLOUR | Increase the other two primaries or decrease its complementary |
| ADJUST GAMMA | Compensate for difference in Brightness between two sources or correct brightness |

COLOUR CORRECTION WHEELS

| COLOUR WHEEL | Red at 11.0 o’clock: Blue at 4.0 o’clock: Green at 7.0 o’clock. |  |  |
| --- | --- | --- | --- |
| LEFT EYEDROPPER | Removes a colour cast by selecting complementary colour: Use for whites, mids etc |  |  |
| RIGHT EYEDROPPER | Adds a colour cast |  |  |
| MOVE TOWARDS GREEN |  | RAISES GREEN | LOWERS RED & BLUE |
| ” ” YELLOW |  | ” RED & GREEN | ” BLUE |
| ” ” RED |  | ” RED | ” BLUE & GREEN |
| ” ” MAGENTA |  | ” RED & BLUE | ” GREEN |
| ” ” BLUE |  | ” BLUE | ” RED & YELLOW |
| ” ” CYAN |  | ” BLUE & GREEN | ” RED |

COLOUR CORRECTION TECHNIQUES

| CHECK EXPOSURE | 1 2 3 4 5 | Convert to greyscale using B & W filter or HSL Saturation Use Levels to adjust exposure Use Waveform: upper limit 100 IRE, lower 0 IRE to avoid/reduce clipping & crushing Set Black at 7.5 IRE & white at 100 IRE (16.5 & 234 RGB) Remove B & W filter |
| --- | --- | --- |
| SET WHITE BALANCE | 1 2 3 | Use Colour Corrector Select white tone (white & high end hues) with Left eyedropper Similarly for mid (facial colours, clothing & almost all else in focus) & dark (shadows & black) tones (not always necessary to use both) |
| USE COLOUR CURVES | 1 2 3 4 5 6 7 8 9 | Horizontal axis = Input or source level: Vertical axis = Output level Lower handle adjusts darker tones (down & right to darken) Upper handle adjusts highlights (up & left to brighten) Steeper the curve the greater the contrast S shape is often ideal To change Gamma: Create point (2 x L cl) in mid-curve and drag up or down To change Black level: 2 x L cl on bottom right corner and drag up (to increase) or right (to reduce) To change Highlight level: 2 x L cl on top right corner and drag down (to reduce) or left (to increase) EXAMPLES: See Hullfish pp 107 – 112 |
| USE COLOUR CORRECTOR | 1 2 3 | See SET WHITE BALANCE as above OR Add/Remove colour cast (see COLOUR CORRECTION WHEELS above) using right/left eyedropper Adjust colour by moving centre point around wheel |
| USE SECONDARY COLOUR CORRECTOR | 1 2 3 4 5 6 7 | Use Select Effect Range Eye Dropper to identify colour to be modified (drag to colour to select) Click Show Mask to identify area selected Use Luminance sliders to define exact area of mask (occasional need to use Saturation & Hue sliders) Add/Remove colour cast (see COLOUR CORRECTION WHEELS above) using right/left eyedropper Adjust colour by moving centre point around wheel Use Saturation & Hue Sliders as or if necessary as well as Gamma, Gain, Offset etc Use new Secondary Colour Corrector for each subsequent colour adjustment |
| USE WAVEFORM | 1 2 | Use RGB Parade Mode to set within 16 – 234 range Use Levels to set blacks/shadows (0 IRE) & Highlights (80 – 100 IRE) |
| USE VECTORSCOPE | 1 2 3 | Use Colour Correction Wheels Correct Blacks and Highlights Select and correct casts with Right Eyedropper by dragging towards complementary |
| USE LEVELS | 1 2 3 4 5 6 5&6 7 | Used to correct colours and brightness (exposure) by matching input and output sources – re-maps total range of the colour(s) Select Colour Channel (“All” is similar to adjusting luminance) Use Input Start/End Sliders to adjust luminosity by amplifying whites and narrowing blacks Use Output Start/End Sliders to adjust luminosity by boosting blacks and narrowing whites If event inside 0 - 100 range use Inputs to boost to fill Waveform graph If event out of range use Outputs to cut to size & then use Inputs if more spread then needed E.G. If Output used to cut whites no Input movement will take it over this limit Use Gamma to compensate for difference in brightness between different sources and to correct brightness in final event (greater than 1 to brighten & less than 1 to darken) |
| USE SATURATION | 1 2 3 4 5 6 | Drag Amount Slider to boost or cut saturation levels Drag Centre Slider to set centre of range of colours to be adjusted (to find setting set Spread to low level, increase Amount setting & drag Centre Slider to find required range) Drag Spread Slider to set range of Hues to be adjusted around centre value Drag Low Slider to boost or cut Saturation from low end of spread to left end of graph Drag High Slider to boost or cut Saturation from high end of spread to right end of graph Use Gamma to adjust for brightness differences between sources |
| USE HSL | 1 2 3 | Hue changes colours in image (each end of Slider leaves hue unaltered) Saturation controls intensity of selected Hue (as in 1). Move all way to left (to 0) removes all Hue to show as B & W Luminance controls overall brightness (value of 1,000 leaves at original level) |
| USE COLOUR GRADIENT | 1 2 3 | Change tones (warm or cool) (warm appears to flatten and cool to increase contrast) Use like a filter Use for masks or fades (useful in titles) |
| USE COLOUR BALANCE | 1 | To adjust Primaries overall or separately in Highlights, Mid-Tones or Shadows |
| MATCH SCENES | 1 2 3 4 5 6 7 | Select master event (preferably a wide angle shot for range of colours) Ensure master is as required for White Balance (see above) and for legal Colours (use Levels and Waveform for shadows & highlights) and save to clipboard Select event for correction & show in Preview Split Preview screen to show both events (i.e. including event on clipboard) (positions can be adjusted) Correct new event as necessary (for White Balance, Colours & Casts etc) (by eye or with Waveform etc): Colour Correction for colour and Levels or Colour Curves for Gamma Use Saturation, Gain and Gamma as necessary If necessary work with both events in Black & White to start with for setting levels |
| DO NOT USE BRIGHTNESS/CONTRAST PLUG-IN UNLESS ABSOLUTELY NECESSARY |  |  |